PROJECT REPORT

Project Title:

Design and Development of Efficient Road object Detection Methods in Extreme Weather  conditions for Autonomous Driving.

1. Introduction

Reliable object detection on roads is foundational to the safety and functionality of  autonomous vehicles. Adverse weather—such as rain, snow, smoke, and fog—degrades the  performance of vision-based detection systems. This project focuses on using deep learning,  specifically Pix2Pix Conditional Generative Adversarial Networks (GANs), to proactively  remove weather-based artifacts from images before passing them to advanced object  detectors like YOLOv5. The solution is evaluated both on high-performance systems and in  resource-constrained environments like Raspberry Pi.

2. Rain Removal using Pix2Pix GAN:

Objective

Develop a deep learning model to automatically remove raindrops from road images,  improving clarity for downstream perception tasks in autonomous driving and surveillance.

Dataset: Trine.Zip – GitHub.

Dataset:https://drive.google.com/file/d/14MJL8FKHobSO8vNLtfj_I80Wc1gGToc6/vie w?usp=sharing

Colab:https://colab.research.google.com/drive/1R3lTlOZpIW7eTKE_RiS0vdrICtLuw Mb8?usp=sharing

 Total Pairs: 861 (rainy/clean)

 Format: 256×256 PNG, paired by ID

 Folders: Rainy: data, Clean (GT): gt/, Pix2Pix-ready: input/ and target/   Integrity: All image pairs

Model Architecture:

 Framework: PyTorch

 GAN Type: Pix2Pix (Conditional GAN)

 Generator: U-Net with skip connections

 Discriminator: PatchGAN (patch-level binary classifier)

Training Setup:

 Epochs: 50

 Batch Size: 4

 Optimizer: Adam (lr=2e-4, β=(0.5, 0.999))

 Device: GPU (CUDA)

 Result Model: pix2pix_rain_50epochs.pth

Output Images:

Performance :

L1 Loss: 0.0373 (low pixel-wise error)

PSNR: 24.92 dB (high perceptual quality)

Conclusion:

Pix2Pix GAN effectively removes rain artifacts, producing clear images ideal for pre processing in autonomous driving perception pipelines.

3. Snow Removal using Pix2Pix GAN

Dataset : Snow100K Dataset – GitHub.

Link:

https://drive.google.com/file/d/103H_nHOAc_dTzOGIcTiZXbB__B1A0F_2/view?usp =sharing

Colab:https://colab.research.google.com/drive/1ND7ittCBADcEHivW0EsinJ91IAbjwlf 0?usp=sharing

 Training: 2500 paired images (train2500)

 Testing: 20 paired images (test2000)

 Size: All images resized to 256×256, horizontally concatenated for Pix2Pix input

Model & Training

 Generator: U-Net with downsampling/upsampling

 Discriminator PatchGAN

 Loss is GAN Loss (BCE) + 100 × L1 Loss

Training:

 Epochs: 100

 Batch Size: 8

 Optimizer: Adam (lr=0.0002, β1=0.5)

 Device: PyTorch (Colab), auto-detection of CUDA/CPUOutput Images:

Evaluation

Average PSNR: 23.07 dB

Average SSIM: 0.7699

Conclusion:

Achieved strong visual improvement and structural fidelity in cleared images. Restored  images are well-suited for computer vision under snowy conditions.

4. Smoke Removal using Pix2Pix GAN

Dataset : SMOKE.zip

Link : https://drive.google.com/file/d/1HevtcMrK-GYLW9E_a2sC-kl8enFy13PG/view

Google Colab Note Book:

https://colab.research.google.com/drive/1_xKP9wDP2x9KkrbG5PYvW7D_IH8z5Uae?usp=s haring

 Pairs: Hazy input / clean GT, 256×256

 Folders: hazy/ and clean/

Model & Training

 Generator: U-Net (6 downsampling/5 upsampling, Tanh out)   Discriminator: PatchGAN (6-ch input, patchwise prediction)

 Losses: GAN (BCEWithLogits), L1 (weighted ×100)

 Training: 100 epochs, batch size 4, optimized with Adam (lr=2e-4)  Output Images:

Results

PSNR: 24.00 dB

SSIM: 0.7646

Conclusion:

GAN robustly cleans smoke-affected images, preserving structure and visual clarity,  and is ready for real-time deployment in driving vision systems.

5. Fog Removal using Pix2Pix GAN

Dataset : Foggy Cityscapes Dataset

Dataset:https://drive.google.com/file/d/1pahlq9sI

gEoih7WC8NP18vkyjvETxdt/view?usp=sharing

Colab link :

https://colab.research.google.com/drive/1x19feXE3qr5qWnwTP0dz0qQe5QHWHhW v?usp=sharing

 Pairs: 450 (training), 50 (validation)

 (Dense_Fog/, Medium_Fog/, No_Fog/)

Architecture & Training

 Pix2Pix GAN: U-Net generator, PatchGAN discriminator

Setup:

 Epochs: 50

 Input: 256×256

 Optimizer: Adam (lr=2e-4)

 GPU-enabled

Output Images:

Results

Outcome: Successfully restored sharpness and detail in urban foggy scenes.  Enhanced clarity supports improved object detection downstream.

6. Weather Removal on Raspberry Pi: Real-World Deployment  A. Snow Removal

 Platform: Raspberry Pi 4B (4GB RAM, gan_env venv)

 Tested Model: Pix2Pix GAN (100 epochs)

 Sample Results:

Image PSNR (dB) SSIM

city_read_05321.jpg 22.88 0.7370

city_read_04087.jpg 22.98 0.7663

| Average | 23.43 | 0.7516 |
| --- | --- | --- |

OutPut Images :

Observations: Pi inference completed in seconds; snow artifacts removed, revealing road  details.

Snow Files :

https://drive.google.com/drive/folders/1oLifbesfJ0wqVysPD37JrkXOtgLt6nAg?usp=sharing

B. Rain Removal

 Platform Raspberry Pi (Python 3.11, PyTorch)

 Dataset: 20 pairs, 256×256

 Training: 5 epochs (to allow quick testing)

Rain Files :

https://drive.google.com/drive/folders/1YLkbeMyX4j4FlmDGAIK8sZfHrHPccStI?usp=sharing OutPut Image :

Results:

 PSNR: ~11.3 dB

 SSIM: ~0.44

Interpretation:

Rapid Pi tests show the GAN learns to reduce rain, though accuracy is limited by few epochs  and small data size. More epochs + larger datasets recommended for production.

7. YOLOv5 Object Detection with Enhanced Images.

Objective

Train a robust YOLOv5-based model to detect road objects after pre-processing  images through weather artifact removal GANs.

Dataset :

Colab :

https://colab.research.google.com/drive/1hCU7C1axFpaImtCh_i0OqQqsHWBFy3he?usp=sh aring

 Validation Images: 479

 Total Instances: 3127

 Classes: Biker, Car, Pedestrian, Truck, Traffic Lights (Green, Red, Yellow,  directionals)

Model & Training

 Detector: YOLOv5s (Ultralytics, small variant

 Framework: PyTorch

 Input: 640×640

 Epochs: 50

 AMP: Mixed-precision enabled

Metric Value

Training Time 1.592 Hour

Best Model Pah runs/train/my_model/weights/best.pt  Box Loss (Last Epoches) 0.0338

Objenctness Loss 0.0304

| Classification Loss | 0.0040 |
| --- | --- |

OutPut Images :

Conclusion:

With GAN-cleaned images as input, YOLOv5 achieves strong accuracy and generalization,  demonstrating readiness for multi-class detection in challenging real-world conditions.

8. Overall Conclusions

GAN-powered image reconstruction delivers substantial improvements to scene clarity under  rain, snow, smoke, and fog.

Embedded feasibility is demonstrated through successful Pi deployments for snow and rain  removal, though training/inference capacity remains a key design consideration.

YOLOv5 object detector performs significantly better on GAN-processed clean images than  weather-distorted ones, making this pipeline robust and effective for autonomous driving  tasks.

Future enhancements:

Real-time Pi camera integration.

Model optimization (quantization, pruning) for faster inference.

Training GANs on larger, more diverse datasets.

Complete end-to-end integration of GAN + detector pipeline for field testing  By

VNKUDOTH PRASHANTH

COMPUTER ENGINEERING

NATIONAL INSTITUTE OF ADVANCED MANUFCTURING TECHNOLOGY RANCHI